import numpy as np

import matplotlib.pyplot as plt

import os

# Set fonts

plt.rcParams['font.family'] = 'SimSun'  # SimSun for Chinese (can be removed if not needed)

plt.rcParams['axes.unicode_minus'] = False  # Handle minus sign

plt.rcParams['font.sans-serif'] = ['Times New Roman']  # Times New Roman for English

# Raw material production data (mean, std, carbon factor)

materials = {

'Cement': {'mean': 459.81, 'std': 28.76, 'cf': 785 * 0.001},

'Water': {'mean': 158.35, 'std': 12.77, 'cf': 0.148 * 0.001},

'Sand': {'mean': 686.32, 'std': 56.61, 'cf': 3.984 * 0.001},

'Gravel': {'mean': 966.94, 'std': 56.94, 'cf': 2.18 * 0.001},

'FlyAsh': {'mean': 45.48, 'std': 21.13, 'cf': 34.5 * 0.001},

'Slag': {'mean': 55.71, 'std': 30.88, 'cf': 62.35 * 0.001},

'Additive': {'mean': 13.75, 'std': 2.66, 'cf': 28.94 * 0.001}

}

# Transportation distance data (mean, std)

distances = {

'Cement': {'mean': 85.46, 'std': 121.15},

'Water': {'mean': 0, 'std': 0},

'Sand': {'mean': 71.92, 'std': 91.63},

'Gravel': {'mean': 55.12, 'std': 77.43},

'FlyAsh': {'mean': 69.23, 'std': 59.87},

'Slag': {'mean': 100.92, 'std': 65.13},

'Additive': {'mean': 282.15, 'std': 473.58}

}

# Transportation mode ratios

transportation = {

'Cement': {'Road': 0.90, 'Rail': 0.07, 'Water': 0.03},

'Water': {'Road': 1.0},

'Sand': {'Road': 1.0},

'Gravel': {'Road': 1.0},

'FlyAsh': {'Road': 0.94, 'Rail': 0.06},

'Slag': {'Road': 0.94, 'Rail': 0.06},

'Additive': {'Road': 0.62, 'Rail': 0.38}

}

# Transportation carbon factors (kg CO2e/kg·km)

transport_factors = {

'Road': 0.078 * 0.001,

'Rail': 0.010 * 0.001,

'Water': 0.019 * 0.001

}

# Electricity consumption data (mean, std)

electricity = {'mean': 2.1718, 'std': 1.0761}

# Electricity carbon factor (kg CO2/kWh)

electricity_carbon_factor = 0.908 * 0.001

# Number of Monte Carlo simulations

n_simulations = 10000

# Function to calculate carbon emission

def calculate_carbon_emission():

# Production emissions of raw materials

total_production_emission = sum(

np.random.normal(data['mean'], data['std']) * data['cf']

for data in materials.values()

)

# Transportation emissions of raw materials

total_transport_emission = 0

for material, data in distances.items():

distance = np.random.normal(data['mean'], data['std'])

for mode, ratio in transportation[material].items():

tf = transport_factors[mode]

amount = np.random.normal(materials[material]['mean'], materials[material]['std'])

total_transport_emission += amount * ratio * tf * distance

# Concrete preparation stage emissions

electricity_consumption = np.random.normal(electricity['mean'], electricity['std'])

total_preparation_emission = electricity_consumption * electricity_carbon_factor

return total_production_emission + total_transport_emission + total_preparation_emission

# Monte Carlo simulation

emissions = [calculate_carbon_emission() for _ in range(n_simulations)]

# Calculate mean and standard deviation

mean_emission = np.mean(emissions)

std_emission = np.std(emissions)

# Plot probability distribution

plt.figure(figsize=(10, 6))

plt.hist(emissions, bins=50, color='skyblue', edgecolor='black')

# Remove x-axis label, keep y-axis label

plt.ylabel('Frequency', fontsize=24, family='Times New Roman')

# Set axis tick font size

plt.xticks(fontsize=22)

plt.yticks(fontsize=22)

# Set x-axis range

plt.xlim(100, 500)

# Display mean and standard deviation on the plot

plt.text(408, 600,

f"Mean = {mean_emission:.2f}\nStd = {std_emission:.2f}",

fontsize=20, family='Times New Roman',

ha='left', va='top',

bbox=dict(facecolor='white', alpha=0.7, edgecolor='black'))

plt.grid(True)

# Save figure to desktop

desktop_path = os.path.expanduser("~/Desktop")

save_path = os.path.join(desktop_path, "carbon_emission_distribution.png")

plt.savefig(save_path)

# Show the figure

plt.show()

# Output basic statistics

print(f"Mean Carbon Emission: {mean_emission:.2f} kg CO2e")

print(f"Standard Deviation: {std_emission:.2f} kg CO2e")